IP Driver Manual for the

Turbo uPSD 3300 Family Microcontrollers

Ver 1.1

April 23, 2004

Introduction

All the drivers provided for the Turbo uPSD have a common format that consists of a header file (.h) and a C file (.c) that make up the specific IP driver.  For example, the ADC IP has two files (Turbo_ADC.c) and (Turbo_ADC.h).  The header file contains the various defines and function prototype statements for the ADC code located in the C file.

All the IP modules make use of two other files that MUST be used (included) in order for the code to compile correctly.  These files are the upsd3300.h file and the TurboLite_hardware.h file.  The upsd3300.h file is the main define file that is used to define the common labels used for the 8032 SFR’s and the various PSD configuration registers.  The contents of this file should NOT be changed.  Please review this file to become familiar with the common labels used.

The TurboLite_hardware.h file defines the labels and values for items that are unique to each hardware design.  To match the specifics of their unique hardware design, the user MUST modify these items.  This includes items such as the oscillator frequency and the CSIOP base address for the PSD registers, etc.

Also included is the STARTUP33.a51 file that must be used to initialize the uPSD’s BUSCON register for proper operation of the device based on the frequency of operation and the operational voltage version.  This startup file is provided as part of the device drivers.  After the initialization, the start up file will jump to the beginning of the C code.

To use one or more of the IP drivers, include the upsd3300.h and TurboLite_hardware.h files as well as the specific IP driver header file in the file that calls the specific IP driver functions.  The IP driver C file is part of the normal build (or makefile) process.  The linker will link the IP driver C object code and the application code together.

Analog to Digital Converter (Turbo_ADC.c)

This driver is used to setup and read the 10-bit ADC unit in the Turbo uPSD.  The driver will automatically set up the prescaler to generate an internal value close to 8 MHz based upon the defined value of the oscillator frequency found in TurboLite_hardware.h.  The driver will also put the selected port1 pins to the alternate function mode.  Calls to the ADC_Read function will return the result after the ADC conversion.

void Turbo_ADC_Init(unsigned channel)

This function must be called first to setup the I/O port (P1.0 –P1.7) to the Alternate Function for the selected ADC channel, as well as  sets up the ADC prescaler value.  This function must be called for each channel setup.

channel – unsigned char value in the range from 0 to 7 that is used to setup ADC channels 0 to 7.

unsigned int uPSD_ADC_Read(channel)

Reads an analog signal from the specified channel specified in hex format, and returns the converted value in the temp_ADC_result variable.  The routine waits for the conversion to complete (approx 10us).   The uPSD_ADC_Init function must be called first to setup the appropriate channels.

channel – unsigned char value in the range from 0 to 7 that is used to read ADC channel 0 to 7.

Programmable Counter Array with Pulse Width Modulation

(Turbo_PCA-PWM.c)

This driver will configure PCA’s timer counter modules (TCM) to PWM mode. There are three PWM configure options:

Mode 1,  8-bit fixed frequency mode.

Mode 2,  8-bit variable frequency mode.

Mode 3, 16-bit fixed frequency mode.

void PWM_Mode1_Init(unsigned char channel, unsigned char duty_cycle)

This function must be called first to setup TCM mode register to 8-bit fixed frequency for the selected  PWM channel. Setup the I/O port (P4.0 – P4.5) to the Alternate Function and setup the PCA Control registers for the select PWM channel.

channel 	- The selected PWM channel number (0 to 5).

duty_cycle 	- The desired duty cycle for the selected PWM channel

void PWM_Mode2_Init( )

This function must be called first to setup the TCM mode register to 8-bit variable frequency, setup the I/O port (P4.0 – P4.5) to the Alternate Function for the selected  PWM channel., setup the PCA Control registers, TCM duty cycle, PWM frequency ratio.

void PWM_Mode3_Init(unsigned char channel, unsigned int duty_cycle)

This function must be called first to setup TCM mode register to 16-bit fixed frequency for the selected  PWM channel,  setup the I/O port (P4.0 – P4.5) to the Alternate Function, and setup the PCA Control registers for the select PWM channel.

channel 	- The selected PWM channel number (0 to 5).

duty_cycle 	- The desired duty cycle for the selected PWM channel.

Timer (Turbo_timer.c)

This driver provides initialization and delay functions using Timer 0.

void timer0_init (void)

This function enables TIMER 0.  TIMER 0 will generate a synchronous interrupt

once every 100Hz.

unsigned int timer0_count (void)

This function returns the current Timer 0 tick count.

void timer0_delay (unsigned int count)

This is a delay function that waits for the specified number of timer 0 ticks to

pass before returning.

count	- unsigned int

- the number of timer ticks to wait before returning from function.

void delay_10ms(void)

This is a 10 ms delay function.

void delay_1sec(void)

This is a 1 second delay function.

void delay_2sec(void)

This is a 2 second delay function.

void delay_10sec(void)

This is a 10 second delay function.

void delay_0_5sec(void);

This is a 0.5 second delay function.

void delay_0_1sec(void);

This is a 0.1 second delay function.

I2C (Turbo_I2C.c)

This driver initializes the I2C port and provides transmit and receive functions for master and slave modes of operation.

unsigned char upsd_i2c_Timeout(unsigned char operation, unsigned int Max_time)

This function is used to indicate if the current I2C operation timed out.

operation 	- unsigned char

- current I2C operation (I2C_MX/I2C_MR/I2C_SX/I2C_SR).

Max_time	- unsigned int

- the wait time.

Returns:

0:	Did not time out

1:	Timed out

unsigned char upsd_i2c_init (unsigned int Bus_Freq, unsigned char Slave_Addr)

This function is used to initialize the I2C port including the specified bus

frequency and slave address.  This function must be called before using the

I2C driver functions.

Slave_Addr 	- unsigned char

- I2C port slave address

Bus_Freq	- unsigned int

- I2C bus frequency (KHz).

Returns:

0:	Initialization successful

1:	Input argument error

unsigned char upsd_i2c_Master_Xmit (unsigned char Slave_Addr,

unsigned char* Data_Ptr,

unsigned char N)

This function is used to transmit a specified amount of data to a slave

while in Master mode.

Slave_Addr 	- unsigned char

- slave address of I2C device.

Data_Ptr	- unsigned char*

- pointer to transimt buffer

N		- unsigned char

- the length of data to be transmited

Returns:

I2C_MX_END 		4			//Master Mode - transmission complete

I2C_TIME_OUT	9			//I2C Timed Out

I2C_NACK		13			//I2C NACK (No Acknowledge)

I2C_BUSLOST		14			//I2C bus lost

unsigned char upsd_i2c_Master_Recv (unsigned char Slave_Addr,

unsigned char* Data_Ptr,

unsigned char N)

This function is used to receive a specified length of data from a slave,

while in Master mode.

Slave_Addr 	- unsigned char

- slave address of I2C device.

Data_Ptr	- unsigned char*

- address pointer of receive buffer

N		- unsigned char

- length of the receive data buffer.

Returns:

I2C_MR_END		8			//Master Mode - reception complete

I2C_TIME_OUT	9			//I2C Timed Out

I2C_NACK		13			//I2C NACK (No Acknowledge)

I2C_BUSLOST		14			//I2C bus lost

unsigned char upsd_i2c_slave_Xmit(unsigned char* Data_Ptr,

unsigned char* N_Ptr)

This function is used to transmit data to master, only for slave.

Data_Ptr	- unsigned char*

- address pointer of transmit buffer

N_Ptr		- unsigned char*

- the length of data buffer transmitted to master.

Returns:

I2C_SX_END		6			//Slave Mode - transmission is complete

I2C_TIME_OUT	9			//I2C Timed Out

I2C_NACK		13			//I2C NACK (No Acknowledge)

I2C_BUSLOST		14			//I2C bus lost

unsigned char upsd_i2c_slave_Recv(unsigned char* Data_Ptr,

unsigned char* N_Ptr)

This function is used to receive data from a master while in slave mode.

Data_Ptr	- unsigned char*

- address pointer to receive buffer

N_Ptr		- unsigned char*

- the length of received data from the master.

Returns:

I2C_TIME_OUT	9			//I2C Timed Out

I2C_SR_END		11			//Slave Mode - reception complete

I2C_NACK		13			//I2C NACK (No Acknowledge)

I2C_BUSLOST		14			//I2C bus lost

I2C_SX_APP		15			//Slave to transmit data to master

LCD (Turbo_LCD.c)

This driver initializes the LCD module, provides a basic printf function to display information on the LCD, and a few other utilities used in converting data for display on the LCD.

void lcd_init(void)

This function is used to initialize the LCD module per the specs.

void BusyCheck(void)

This function is used to wait until the LCD module's busy flag (BF) is cleared.

void printfLCD(unsigned char*,...)

This function is used to display a string of characters on the LCD display.

chr_ptr 	- unsigned char*

- pointer to string to display on LCD.

void lcd_clear(void)

This function is used to clear the LCD.

void MoveCursor(char m_direction, unsigned char m_step)

This function is used to move the cursor on the LCD.

m_direction	- char

- the direction to move the cursor from the current location.

m_step	- unsigned char

- the number of character positions to the move the cursor in the

direction specified by m_direction.

char htoa_lo(unsigned char byte)

This function is used to convert the low nibble of an unsigned byte (0-F hex)

to ASCII.

byte	- byte

- byte to convert to ASCII.

Returns:

ASCII value result of the conversion.

char htoa_hi(unsigned char byte);

This function is used to convert the hi nibble of an unsigned byte (0-F hex)

to ASCII.

byte 	- byte

- byte to convert to ASCII.

Returns:

ASCII value result of the conversion.

Disclaimer

This example demo code is provided as is and has no warranty,

implied or otherwise.  You are free to use/modify any of the provided

code at your own risk in your applications with the expressed limitation of liability (see below) so long as your product using the code contains at least one or more uPSD products (device).

LIMITATION OF LIABILITY:   NEITHER STMicroelectronics NOR ITS VENDORS OR AGENTS SHALL BE LIABLE FOR ANY LOSS OF PROFITS, LOSS OF USE, LOSS OF DATA,INTERRUPTION OF BUSINESS, NOR FOR INDIRECT, SPECIAL, INCIDENTAL OR

CONSEQUENTIAL DAMAGES OF ANY KIND WHETHER UNDER THIS AGREEMENT OR

OTHERWISE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.