DALI Tester

USER MANUAL

Release 1.0

Introduction

This test kit has been realized to allow the automatic functional verify of the ST7DALI-EVAL kit.

This test kit is composed of a board that must be connected to the PC with the parallel interface and with the ST7DALI-EVAL using another set of cables supplied with the test kit.

The test board has been designed to avoid the overload of the kit that must be tested.

But in the same time it is necessary to measure a lot of electrical parameters to permit the detection of fault and suggest a possible solution.

Of course only measuring the electrical parameters it is impossible to guarantee the efficiency of the kit, for this reason on the PC is installed a program that send to the ST7DALI-EVAL a set of command to verify the right response of the kit in terms of communication and electrical levels

Test system

The system is composed of a PC interface and an intelligent board of measure.

The testing board has the full control of the ST7DALI-EVAL kit this means that this board can reset each slave or master of the DALI kit.

This capability is useful to permit a logical disconnection of one node from the network during the test if this one doesn’t work, of course a logical disconnection can’t solve the problem of low voltage on the DALI bus if this is due to a short circuit on the DALI interface of the slave node.

The test board is the core of the verify system, as already mentioned it is composed of a parallel connector to permit the connection with the PC, moreover on this board is present a microcontroller  ST52F513.

In this microcontroller are used two peripherals, the I2C to communicate with the PC and the A/D converter to permit the measure of the electrical parameters.

In fact, an important feature of this micro is the availability of an A/D converter multiplexed on 8 channels.

Unfortunately for this application we need 10 channels, so to reach this target is used also an analog multiplexer, also made by ST the M74HC4016B1.

On this board are available also:

- 3 ISP connector with the standard of  MCD,  to connect the test board with the correspondent ISP connector in the master and slaves boards. (1 Master and 2 slaves)

- One connector to connect the test board with the DALI bus using the supplied cable.

- Two connectors to connect the test board with the 1..10V standard output of the slaves nodes.

Of course the working range of the A/D converter is from 0 to 5V, this means that all the parameters, inside this range, are directly connected to one of the converter input, instead of the other ones are managed by a partition resistor to reduce the amplitude.

How to connect the System

To use this tester kit, you have to connect the ST7DALI-EVAL before and then proceed with the connection of the three ISP connectors.

Now you can connect the 1..10V ballast out with the correspondent connector in the test board, during this operation, but  pay attention to the number of the ISP connector and use the same number of the 1..10V cable.

Done this,  it is possible to connect the DALI bus cable.

After a cross check of the connections, it is possible to connect the ST7DALI-EVAL kit to the PC using the RS232 cable and the DALI Tester kit using the parallel cable.

Now it is possible to supply the kits and run the DALI Tester PC interface.

In the next figure (figure 1) is showed the right connection way.

Figure 1.

ISP Master

ISP Slave 0

ISP Slave 1

OUT 1

OUT 0

BUS      DALI
I

Parallel Port      DAL
I

Serial Port      DAL
I

TO PC

TO PC

ISP Slave 0

ISP Slave 1

ISP Master

9VDC

PC Graphic interface:

This program allows the complete verify of the kit in an easy way.

It appears as showed in the figure 2.

Figure 2

The interface is composed of a menu bar that permits to the user many operations.

The descriptions of each item are the follows:

The submenu File and Help are the simplest because they contain only one field, in particular, the file submenu allows the closedown of the program. Instead in the help submenu is possible to show the information about the writer of the program and where to send an E-mail for information about the product.

The View submenu enables an echo window where it is possible to see the DALI commands sent from the master to the slaves and the slaves’ answers with the right DALI name.

The Option submenu is divided in two submenus: in the first part it permits the selection of the working modality in particular ‘user mode’ and ‘test mode’. In the second one, the user can select the slave that must be checked; of course at least one slave must be selected.

It’s important to note that this disconnection is only a logical disconnection; this means that if there is a problem in the DALI interface of the slave X deselecting the slave X won’t avoid the issue. It will work correctly for the other problems.

User mode :

In user mode, the graphic interface permits to the user the slave selection and the run of the kit test.

The kit test works as follow:

- It resets the ST7DALI-EVAL kit and verifies the right functioning of the all leds of the kit, asking to the user if they are ON.

- It executes the Electrical test, this means to verify  the voltage of the DALI bus,of the DC bus and all the VDD and reset Voltage on the ST7DALI-EVAL boards. The results of this test are showed in the top part of the graphic interface, with the real measured value of each parameter.

- It executes the Communication test; with this one the PC configures the DALI network with the standard algorithm of the DALI protocol. This test usually takes about 20 seconds. In the communication part, it is possible to know the number of slaves detected and the status of the communication test and when it will be completed the result.

- The last test is the ballast test, this permits to verify the 1..10V interface. In fact, during this test it will be sent to the slave three different values of light dimming values and will be verified the right value of the output voltage value. Also in this case it is possible to know the status of the test looking to the Ballast OUT test part.

Test mode:

In this test modality the user can execute all the possible tests in whatever order.In fact, it will be available a lot of test buttons.

In particular, for the electrical test the user can change the threshold of the voltage that must be verified.

In this modality it is available to the user a button to switch ON or OFF the DALI Tester board. It is possible to know the status of the board looking the green led.

This modality is suggest only to expert users and only if they want to execute a deep test.

In the figure 3 you can see the graphic interface in this modality.

Figure 3.

Message handler

The error messages and the test results are showed in a special dialog window.

In the figure 4 you can see this dialog window.

Figure 4.

In the first three rows are reported for each test, the status that can be:

- No exec, this means that this test was not executed.

- Failed, means that a problem occurs. In this case in the next rows you’ll find the problem that happened and a possible solution.

- Test OK, this means no problem detected.

In the next rows of the message window you’ll find a list of the errors occurred with a short description, enabling the details button is possible to have a long description, in this way is possible to have an additional help in the error detection and solving.

Errors list.

In this paragraph, is reported the description of the possible errors with the short description and a detailed problem management.

cod. 0

short description: Low voltage on DALI BUS

Details: This problem can be due to:

- If this value is higher than 22V then verify the supply of the test board.

- No supply to the kit

- If the value is about 3V then disconnect the connector J1 and verify:

  - Run Again the test, if the problem persist then go to the next points

  - Transistor Q1

  - Optoisolator U3 if the voltage on pin2=3.5V then change the microcontroller

  - Optoisolator U3 if the voltage on the base pin of Q1=0.7V then change U3

  - Optoisolator U3 if the voltage on pin 4 of the opto is 0 then C14 is a shortcircuit

  - If the voltage on pin 2 of the opto U4 is 0 then D5 is a shortcircuit

cod. 1

short description: Low voltage on DC BUS

Details: This problem can be due to:

- No supply to the kit

- Diode D6 broken

  - Verify the drop voltage

- Voltage regulator U5

- Capacitor C9

  - Try to disconnect it

cod. 2

short description: Low voltage on VDD master

Details: This problem can be due to:

- Connection of the J4 connector (ISP) with the Test board

- Voltage regulator U6

- Capacitor C11, C4, C5, C3

  - Try to disconnect in this order C11, C4, C5, C3

cod. 3

short description: Low voltage on master RESET pin

Details: This problem can be due to:

- Bad connection of the J4 connector (ISP) with the Test board

- C13 is a short-circuit

- Try to disconnect it

cod. 4,6

short description: Low voltage on VDD slave X

Details: This problem can be due to:

- Bad connection of the J3 connector (ISP) with the Test board

- Bad connection of the jumper j4

- Voltage regulator STLD2980 may be broke

- Capacitor C2, C3, C4

  - Try to disconnect in this order C2, C3, C4

cod. 5,7

short description: Low voltage on RESET slave X

Details: This problem can be due to:

- Bad connection of the J4 connector (ISP) with the Test board

- capacitor C6

- Try to disconnect it

cod. 8,9

short description: Ballast out slave X

Details: This problem can be due to:

- Bad connection with between connector j2 and Test board

- Connection of resistor R15

- OP-AMP (STTS271)

  - Voltage on pin 7 of the OP-AMP (STTS271) must be 12V

  - Voltage on pin 3 of the OP-AMP (STTS271) must be half of the pin 6 one

cod. 10,11

short description: Wrong response slave X

Details: This problem must be verified by a technician:

cod. 12,13

short description: No response slave X

Details: This problem can be due to:

- if the voltage on pin 4 of the bridge D4 is less then 16V substitute D4

- the OPTO U4

- the transistor Q1

cod. 14

short description: Wrong test board response

Details: Verify the connection of the parallel cable

cod. 15

short description: Master Board did not respond!

Details: To verify:

- The Supply of the master board

- The insertion of the RS-232 cable

- The ST232 level shifter

  - Verify the continuity between the microcontroller and the level shifter

    - Micro pin 37 ST232 pin 11

    - Micro pin 38 ST232 pin 12

  - ST232 supply

    - +5V on pin 16

    - ST232 supply Ground on pin 15

  - Verify the continuity between the level shifter and the RS232 connector

    - ST232 pin 14 connector pin 2

    - ST232 pin 13 connector pin 3

cod. 16

short description: NO all slave detected

Details: This problem can be due to:

- one slave isn’t connected

- problem in the communication interface of the not detected slave

cod. 19

short description: NO test board FOUND

Details: Check the supply of the test board

cod. 20

short description: NO slave found

Details: This problem can be due to:

- no slaves connected

cod. 23

short description: Not all leds ON

Details: This problem can be due to:

- Soldering problems

- Microcontroller problem

- Verify the potential of the correspondent microcontroller pin, it must be 0V