Hi Brandon,

For Char LCD:

LCD_RS/RW/E 2.5V bank signals go through CPLD for level translation.

LCD_DB[4:7] Use 3.3V I/O bank and run signals through CPLD for level translation.

Remove 74CBTD3384 part since we are using CPLD for level conversion.

For LCD contrast POT, Add 6.81K R between Vcc5 and pin 3 of Pot.

Change Pot to 2K single turn thumb pot with following part num/manufacturer

| 3352W-1-202LF | Bourns |
| --- | --- |

Hi,

Here is the Piezo info:

Part#/Manufacturer:

| AT-1220-TT-2-R | Projects Unlimited |
| --- | --- |

Datasheet http://web/seg/ml50x/datasheets/AT-1220-TT-2-R.pdf

For the piezo, please use this circuit. Replace transistor with FET. VDC=3.3V

For protection, please add series R of 100 ohm between VDC and Piezo.

-So no DDR diff term resistor at the FPGA, we can use internal resistor.

-no 50 ohm parallel term resistors to VTT on  DQ/DM/DQS since we use DCI.

-add a 5 pf 0402 cap at the clock inputs of the DIMM

-no clock feedback needed for DDR2 interfaces

-no vtt term R’s on ODT and CKE, only 4.7K pulldown as per recommendations

-for now leave CKE pins separate. If we run out of pins can tie together.

-Khang

Hi Brandon,

Looking at these spec, this is what is implied:

3 banks for data pins with vrefs and vrn/vrp used. 2 banks have 24 bits of data, 1 bank has 16 bits of data.

1 bank for control with no vrn/vrp and no vrefs (outputs only), split term at sodimm side

Spread out CK, ODT, CKE over 4 banks where there is space.

Fill in rest of 1.8v banks with priority list I previously send you.

Assign pins to ddr2 signals over 4 banks to optimize layout then throw in remaining low speed 1.8v signals like gpio buttons.

-Khang

Hi Brandon,

For passives, all resistors currently 0603 will be 0402 sized. Some resistors on ML405 bigger than 0603 stay same size for power reasons.

For caps I’m doing research into sizes vs values and will have guidelines….

-Khangh

Hi,

For audio circuit:

ML501 use Kycon STX-3500-3N (SMT)

For ML505 use SMT as well except headphone out connector which uses through hole Kycon STX-3150-3N for mechanical reasons.

RCA connector for SPDIF is digikey:

| RCJ-021 | CUI Inc |
| --- | --- |

See attached for datasheets

-Khang

Hi Brandon.

The piezo goes on the 1.8V bank with higher priority that LEDs.

-Khang

Hi,

We will use same transistor as AFX daughtercard for all ML50X boards:

Digikey NDS331NTR-ND

Datasheet = http://www.fairchildsemi.com/ds/ND/NDS331N.pdf

Seems to have same pinout and decal as the current BSS138 so just update symbol attributes.

-Khang

Hi,

OK I thought it over and this is what I think we should do:

1) Create fourth bank for extra DDR signals. Put DDR control outputs only on 4th bank (ODT, CS, CKE, upper address/BA bits that are only in the larget SODIMMs. Therefore no vrefs/ no ddr loop/no DCI is needed on the fourth 1.8V DDR bank.

2) Full out remaining pins on 1.8V DDR bank with:

a. Dip switches (8)

b. Push buttons, NESWC+CPU reset (6)

c. PS/2 pins (4)

d. IIC main bus (2)

e. IIC VGA bus (2)

f. LEDs (15). Each LED output drives a transistor which in turn drives the LEDs.

3) Then in the expansion bank with 12 pins left, place the Ethernet PHY signals. Enet PHY signs are 2.5V with 3.3V tolerant I/O so whether expansion is 2.5 or 3.3V it still works.

4) In the 2.5V clock bank place (No DCI on this bank):

a. LVDS in/out (4)

b. Main 100 MHz clock which will be LVDS (2)

c. DDR clock feed back pair (2)

d. Marvel Phy receive clock (PHY_RXC_RXCLK) – must put on P side of IOB diff pair

e. LCD control signals RW/RS/E (3) – go through CPLD for 3.3V to 5V conversion

f. Remaining LEDs from #2f. Buffer these LEDs through CPLD

g. Marvel Phy slow speed signals (MDC/MDIO/MDINT)

I will research a lower gate voltage transistor. The BSS138N we use now will not work with 1.8V.

-Khang

Hi,

Make sure you also copy the fan controller from ML405 to ML505.

-Khang

Hi,

Can you make the following Audio Changes:

1) Change LM4550 to Analog Devices AD1981B.

2) Add RCA connector to SPDIF output

3) Change 5 pin audio connectors to 3 pin connectors, removing pin 3 and 4 jack sense pins.

Thanks,

-Khang

Hi,

Attached are the datasheets for socket and DIMM.

We are using socket part number

| MM50-200B2-1E | JAE Electronics |
| --- | --- |

Disti= mouser

PS/2     Mini Din6 TH

Mouse

PS/2     Mini Din6 TH

Keyb

Serial Port DB9M     TH

RS232

PC4   14 Pos SMT

SW SMT

L

L

L

L

DC IN        2.1x5.5mm

TH

16x2 Char LCD TH

L

L

L

L

GPIO LEDs

L

L

PLB/OPB Err

L

L

INIT/ DONE

L

L

L

L

DIP-8 Switch SMT

CPU Reset

Prog Button

FLASH     TSOP56 SMT

L

L

L

L

L

L

PHY LEDs

Marvell ENET PQFP128     SMT

CY7C1354 SRAM TQFP100   SMT

XC9500XL TQ100  SMT

ADI 3202

L

PWR Good

USER XTAL TH

IIC

5VLX30/50/85/110

SMT

SYSMON

SW TH

Piezo TH

SW SMT

SW SMT

SW SMT

SW SMT

SW SMT

SW SMT

L

EZHOST TQFP100 SMT

ADV7125 LQFP48 SMT

CompactFlash         Type I/II   SMT

XCACE TQ144 SMT

SMA TH

SMA TH

SMA TH

SMA TH

SPI Flash Prom

Audio 3.5mm SMT

Audio 3.5mm SMT

Expansion Header     1x32 Pins   TH

Expansion Header     2x32 Pins   TH

Expansion Header     1x32 Pins   TH

Expansion Header     1x32 Pins   TH

Expansion Header     1x32 Pins   TH

AD1981 LQFP48 SMT

USB B Periph TH

USB    A Host   TH

USB

USB

USB B Periph TH

USB

Audio 3.5mm SMT

DIP-6 Switch SMT

3 Pos Switch SMT

DDR2 SDRAM SODIMM SMT

XCF32P VO48 SMT

Audio 3.5mm SMT

Can you print me out the schematic pages with FPGA symbols on them.

Hi,

I think we have to keep them together because of pin counts. We are adding VGA input which is like another 30 pins. We also connect the superclock control to FPGA I/O which is 8 more pins not currently on ML405.

The USB interface is very slow 10-20 MHz but sysace is 33 MHz and they must share databus.

On ML505 usb and sysace are close together.

On ML501, boards are small so it should still work at speed.

Attached is a rough pincount estimate. You can see it is pretty tight and I’m sure I’m missing some misc pins here and there.

-Khang

/home/khangdao/tmp/datasheets

Attached is BOM listing exact part numbers to be used.

Note:

-Video stuff not finalized

-Power Supplies Not Finalized

-Clocking not finalized

The rest is pretty solid.

-Khang